UNIT: UKULELE

INVISIBLE FIELDS, REAL FORCES

GOAL:

Use a magnet and copper wire coil to investigate electromagnetic induction. Can motion or vibration create electrical current without batteries? Let’s find out!

Materials:

- Magnet

- Copper wire coil (or small motor)

- Multimeter

- Ukulele strings

STUDENT DIRECTIONS:

STEP 1: Connect your circuit:

  - Attach the copper wire coil to the multimeter (set it to measure milliamps (mA) of current).

  - Make sure your coil is stable and not moving on its own.

STEP 2: Test magnetic motion:

  - Move the magnet back and forth quickly and steadily near (but not touching) the coil.

  - Watch the multimeter — does it show any current?

  - Try different speeds and directions (slow, fast, circular) and see how the reading changes.

STEP 3: Test string vibration:

  - Place the coil and magnet close to a ukulele string (ideally right beneath or behind the soundboard where vibrations are strongest).

  - Pluck the string — observe the multimeter for any movement in current.

  - Try plucking harder or softer and record your results.

Magnetic Induction Table

| Test Type | Movement Type | Current Observed (mA) |
| --- | --- | --- |
| Magnet only | Back-and-forth motion |  |
| String vibration | Pluck near coil/magnet |  |

REFLECTION QUESTIONS:

### 1. What’s happening to the energy in this system?
→ ___________________________________________________________
(Hint: Where does the energy start? What form does it take?)

### 2. Which action produced more current — the moving magnet or the vibrating string? Why do you think that is?
→ ___________________________________________________________

### 3. How does this experiment show that magnetic fields can cause electric currents?
→ ___________________________________________________________

### 4. If you were designing a generator powered by string vibration, what would you improve about this setup?
→ ___________________________________________________________

WRAP-UP REFLECTION:

- What scientific principle most affected the ukulele’s sound and behavior?
→ ___________________________________________________

- How did changes in force or material affect energy transfer?
→ ___________________________________________________

- If you had to improve this ukulele for precision tuning and durability, what would you change based on your data?
→ ___________________________________________________

EXTENSION IDEA:

Design a custom ukulele body using recycled materials (plastic bottles, cardboard, 3D printed parts). Test how the material affects sound quality, heat response, and electrical output.

### Standards Alignment

### NGSS: HS-PS3-5  STEL: STEL 1E, STEL 2H, STEL 8E, STEL 10F, STEL 11G CCSS: CCSS.MATH.CONTENT.HSN.Q.A.1, CCSS.MATH.CONTENT.HSF.IF.C.7, CCSS.MATH.CONTENT.HSS.ID.C.7, CCSS.MATH.PRACTICE.MP2, CCSS.MATH.PRACTICE.MP4, CCSS.MATH.PRACTICE.MP5