### 📍 Station 3: Energy in Every Vibration

NGSS: HS-PS3-3 – Energy and Its Conservation

### Goal:

Strum a ukulele connected to a piezoelectric pickup and measure the voltage it produces. Compare energy output at different strum strengths and explore how mechanical energy becomes electrical energy.

🛠️ Materials:

- Multimeter

- Piezoelectric pickup (attached to ukulele)

- Oscilloscope app or sound meter (optional)

- Graph paper

Student Directions:

STEP 1: Get your gear ready:

  - Make sure the piezoelectric pickup is securely attached to the ukulele (usually near the bridge or soundboard).

  - Connect the multimeter to the pickup (set it to Volts (V) for DC or AC depending on your pickup type).

  - Optionally, set up a decibel (dB) meter app or oscilloscope app on your phone for extra data.

STEP 2: Test different strum strengths:

  - Strum the ukulele gently and record the voltage output and sound level (if available).

  - Repeat with a medium-strength strum and then a hard strum.

  - Wait a few seconds between each to let the reading stabilize.

STEP 3: Record your data:

  - Fill in the chart below based on your measurements.

### Voltage Output vs. Strum Strength:

| Strum Strength | Voltage (V) | Sound Level (dB) |
| --- | --- | --- |
| Soft |  |  |
| Medium |  |  |
| Hard |  |  |

### Graph It:

Use graph paper or the space below to make a bar graph:

- X-axis = Strum Strength (Soft, Medium, Hard)

- Y-axis = Voltage Output (V)

### 💬 Reflection Questions:

1. What happens to the voltage as you strum harder?
→ ___________________________________________________________

2. How is energy being transferred and transformed in this activity?
→ ___________________________________________________________
(Hint: think about mechanical energy turning into electrical energy!)

3. What does this tell you about energy conservation in vibrating systems?
→ ___________________________________________________________

4. Why might piezo sensors be useful in other real-world technologies (e.g., guitars, shoes, medical devices)?
→ ___________________________________________________________

📍 Station 3: Energy in Every Vibration
NGSS Standard:
HS-PS3-3 – Design, build, and refine a device that works within given constraints to convert one form of energy into another form of energy.

### ✅ ITEEA STEL Standards (Standards for Technological and Engineering Literacy)

STEL 1E – Science and engineering practices can be integrated into the study of technology.
→ Students apply scientific principles of wave energy and electrical output to explore energy conversion through sensors.

STEL 2L – Malfunctions of engineered systems often have predictable or diagnosable causes.
→ Learners observe and interpret sensor behavior based on strum strength and pickup quality.

STEL 8F – Evaluate designs using functional, aesthetic, social, economic, and environmental criteria.
→ Students assess the effectiveness of their piezo setup and consider real-world uses.

STEL 8G – Test and evaluate design solutions using criteria and constraints.
→ By testing strum strength and measuring electrical output, students evaluate energy conversion performance.

STEL 10F – Use measuring instruments to collect data.
→ Students use a multimeter (and optionally, sound level meters or oscilloscope apps) to quantify output.

### ✅ Common Core Math Standards – High School

CCSS.MATH.CONTENT.HSF.IF.C.7 – Graph functions expressed symbolically and show key features of the graph.
→ Students graph voltage output vs. strum strength to visualize patterns and relationships.

CCSS.MATH.CONTENT.HSN.Q.A.1 – Use units as a way to understand problems and guide the solution of multi-step problems.
→ Students correctly measure voltage in volts and interpret numeric data meaningfully.

CCSS.MATH.CONTENT.HSN.Q.A.3 – Choose a level of accuracy appropriate to limitations on measurement.
→ Students decide on the best rounding or precision levels for multimeter readings.

CCSS.MATH.PRACTICE.MP2 – Reason abstractly and quantitatively.
→ Learners interpret and explain what the voltage measurements mean in the context of energy transfer.

CCSS.MATH.PRACTICE.MP4 – Model with mathematics.
→ Students model the energy conversion process and compare physical actions with numeric outcomes.

CCSS.MATH.PRACTICE.MP5 – Use appropriate tools strategically.
→ Strategic use of digital tools (multimeter, graphing, meters) enables data-driven conclusions.

### ✅ Summary

In this energy-focused engineering station, students engage in real-world data collection using piezoelectric sensors and multimeters to explore how mechanical energy from vibration transforms into electrical energy. This bridges physics, electrical engineering, and design, making it a powerful activity for high school students in STEM, CTE, and Biomed pathways.

Top of Form

Tools

Bottom of Form